N1; BILLET SIZE X 115 Y140 Z6

N2 G21 G17 G90 T1 D6 F100 S1000 M03 M06; Metric system. X-y plane, absolute dimensioning, drilling tool, tool diameter, feed rate, spindle speed, spindle on, coolant on

N3 G40; Cancel tool compensation

N4 G00 X55.716 Y19.002 Z6; Rapid traverse to point (i)

N5 G81 X55.716 Y19.002 Z-7 R3; Drill at point (i)

N6 G81 X65.904 Y28.811 Z-7 R3; Drill at point(ii)

N7 G81 X56.095 Y38.998 Z-7 R3; Drill at point(iii)

N8 G81 X45.908 Y28.811 Z-7 R3; Drill at point(iv)

N9 M06 D6 T2 F120 S100; Automatic tool change, endmill cutter, tool diameter, feed rate, s

N10 G42;Compensation right

N11 G00 X17 Y35 Z5    ; Rapid move to start position

N12 G01 Z-3 F100      ; Move to cutting depth

N13 G03 X17 Y35 I17 J22 ; Circular interpolation (counterclockwise)

N14 G01 X17 Y32 Z-3; Move to new cutting position.

N15 G03 X17 Y32 Z-3 I17 J22; Circular interpolation

N16 G01 X17 Y29 Z-3; Move to new cutting position

N17 G03 X17 Y29 Z-3 I17 J22; Circular interpolation

N18 G01 X17 Y26 Z-3 ; Move to smaller radius

N19 G03 X17 Y26 Z-3 I17 J22; Circular interpolation

N20 G01 X17 Y23 Z-3; Move to smaller radius

N21 G03 X17 Y23 Z-3 I17 J22

N22 G01 X17 Y21 Z-3

N23 G03 X17 Y21 Z-3 I17 J22; Circular interpolation

N24 G42; Compensation right

N25 G00 X36.542 Y5 Z6; Rapid traverse to point d

N26 G91; Incremental system

N27 G01 X0 Y0 Z-15; linear traverse

N28 G01 X35 Y0 Z0; linear interpolation to point c

N29 G01 X0 Y-25 Z0; Linear interpolation to point b

N30 G01 X-35 Y0 Z0; Linear interpolation to point a

N31 G01 X0 Y25 Z0; linear interpolation to point d

N32 G01 X0 Y0 Z11; retract tool

N33 G90; Absolute System

N34 G00 X15 Y45 Z6; Rapid traverse to point F

N35 G40; Cancel tool compensation

N36 G41; tool compensation left

N37 G01 X15 Y45 Z-6; Linear traverse at point F

N38 G01 X75.095 Y45 Z-6; Linear interpolating to point G

N39 G01 X75.095 Y20 Z-6; Linear interpolating to point H

N40 G01 X115.095 Y-10 Z-6; Linear interpolating to point I

N41 G01 X115.095 Y-50 Z-6; Linear interpolating to point J

N42 G91; Incremental system

N43 G01 X-20 Y0 Z0; Interpolation to point K

N44 G03 X-40 Y0 Z0 I-20 J0; Circular interpolation to point L

N45 G01 X-60 Y0 Z0; Interpolation to point A

N46 G01 X0 Y20 Z0; Interpolation to point B

N47 G01 X30 Y30 Z0; Interpolation to point C

N48 G01 X-25.095 Y0 Z0; Intepolation to point D

N49 G01 X-5 Y5 Z0; Interpolation to point E

N50 G02 X20 Y40 Z0 I20 J15; Circular interpolation to point F

N51 G00 X0 Y0 Z22; Retract tool

N52 M05; Spindle off

N53 G40; Cancel tool compensation

N54 M09; Coolant off

N55 G28; Machine home

N56 M30; Program end